# NetBridge — verified working configuration (2026-09-24)

Status: verified at 20:34 IST and re-checked at 20:49 IST. The presenter confirmed smooth video, no freezes and no latency issues. Video, voice and return audio were all live, with 0 media-service restarts and 0 lost packets from the Mac to the Pi.

Sources. Every value below was read from one of these:

- the running processes on the Mac and the Pi;

- the live USB gadget (configfs);

- the Pi's own files.

Each value was then cross-checked against the code on branch opt/pi-2026-09-22. Nothing was changed to take these readings.

A Word copy is next to this file: docs/NetBridge-Working-Configuration-2026-09-24.docx.

# 1. Components and versions

| Part | Version / setting |
| --- | --- |
| Pi image | 2.1.0-52a161b: GitHub draft release v2.1.0-52a161b, image sha256 0f316e5067ed3cc9596259053bd24445087232f0835fb84ecca7d5b6b5a60921, kernel 6.12.93+rpt-rpi-v8 (pinned as kernel612.img). It was audited before flashing: tools/image-audit.sh passed 107/107 and the file-by-file verifier passed 54/54. All 930 installed packages are identical to image 8434651 |
| Pi camera service | Signed override 97219e9c at /data/overrides/bridge-uvcd.sh. It is the image's camera script plus a USB-miss counter, with the USB interrupt (IRQ 34) on CPU 2. Since commit 6f8a60a this is also the built-in pi/scripts/bridge-uvcd.sh, so an image built from this branch starts this way without the override |
| Other Pi media scripts | Built in, with no overrides: feeder-net 23cb9bca, feeder-audio c4181446, return-audio 282db308 |
| Mac app | NetBridge 1.4.3, the default app in ~/Desktop/NetBridge. Older builds are in its Older versions folder |
| Mesh | Tailscale, on the direct path over the local Wi-Fi |
| Media profile | WAN (NET_VIDEO_LATENCY=300, NET_AUDIO_LATENCY=300), seeded in /data/config/bridge-net and bind-mounted onto /etc/default/bridge-net. The video feeder caps video at 100 ms |
| Code | Branch opt/pi-2026-09-22 (pushed; not merged into main) |

# 2. Video: Mac camera → Pi → meeting laptop (USB camera)

| Stage | Setting |
| --- | --- |
| Mac capture | MacBook Air camera via AVFoundation (-f avfoundation -framerate 30 -video_size 1280x720 -pixel_format uyvy422). Asking the camera for 320×180 or 640×360 fails outright, so the app captures large and scales down |
| Mac scale | -vf scale=424:240,format=nv12 |
| Mac frame rate | 30 fps constant (-fps_mode cfr -r 30), the camera's native rate. 20 fps was dropped because keeping 2 of every 3 frames spaced them 33/67 ms apart, which showed as judder |
| Mac encoder | h264_videotoolbox -realtime 1 -profile:v baseline, -b:v 600k, -g 30 (one keyframe per second), -bsf:v dump_extra=freq=keyframe (SPS/PPS repeated on every keyframe). Baseline uses CAVLC, which measured 41% less Pi decode CPU than High/CABAC with no visible difference (SSIM 0.966 vs 0.967) |
| Mac transport | RTP payload 96, pkt_size=1100 → 127.0.0.1:5000 → mesh helper netbridge-mesh (1 MiB socket buffers, logs packets/gaps every 10 s) → Pi port 5000 |
| Pi jitter buffer | rtpjitterbuffer latency=100. The WAN profile says 300, but bridge-feeder-net.sh caps video at 100 ms; a non-numeric value falls back to 100 |
| Pi decode | rtph264depay ! h264parse ! avdec_h264 (software). The hardware decoder is deliberately unused (gpu_mem=16): it measured 74% of a core against 51%, because its output conversion cost more than the decode it saved |
| Pi convert | videoconvert ! videoscale ! videorate ! video/x-raw,format=YUY2,width=424,height=240,framerate=30/1. No actual scaling happens |
| Pi hand-off | v4l2sink device=/dev/video40 sync=false → v4l2loopback (video_nr=40 card_label=BridgeCam exclusive_caps=1 max_buffers=16, YUYV:424x240@30) → uvc-gadget pump. The pump re-sends the latest frame when no new one is ready |
| USB camera (UVC) | Uncompressed YUY2 424×240 @ 30 fps (dwFrameInterval 333333), frame buffer 203,520 bytes (dwMaxVideoFrameBufferSize) |
| USB transport | Isochronous, streaming_maxpacket 1024: one 1,024-byte packet per 125 µs microframe, with no high-bandwidth mode, on USB 2.0 high-speed. The stream needs 6.1 MB/s, 75% of the single-packet ceiling of 8.2 MB/s. 640×360@20 needed 9.2 MB/s, which forced 2048 (two packets per slot); that is the mode where the Pi's dwc2 controller missed slots and every meeting app showed freezes |
| USB interrupt | DWC2 IRQ 34 pinned to CPU 2 (USB_IRQ_CPUS="2"). WaysToGo measured 78 -61 misses per 20 s on CPU 0 and 1–5 on CPU 2 (2026-09-16). Setting it to "0-3" restores the kernel default for A/B tests |
| Idle picture | A black 424×240 frame (/etc/bridge/idle-frame.raw, 203,520 bytes), sent by the pump after 5 s without a fresh frame (IDLE_FALLBACK_S 5) |
| Camera logs | The unable to dequeue buffer … (11) EAGAIN flood is filtered out. Pump telemetry every 2 s: pump: ok= again= err= gray= idle= |
| USB-miss counter | Written to /run/netbridge-usb-video.txt. Every 10 s it records enodata (-61), exdev (-18 / missed xfer), other, and the per-CPU USB interrupt load; it keeps the last 360 lines. Read it with the fleet read-file command (section 9) |

# 3. Audio: voice, Mac microphone → Pi → meeting laptop's microphone (unchanged)

| Stage | Setting |
| --- | --- |
| Mac capture | "System default microphone": osxaudiosrc device=0 buffer-time=40000 latency-time=10000 (backend gstreamer-coreaudio). Mic boost 0 dB, not muted |
| Mac format | 48 kHz stereo F32LE, then S16LE |
| Mac encoder | opusenc bitrate=64000 audio-type=voice frame-size=20 inband-fec=true packet-loss-percentage=5 |
| Mac transport | rtpopuspay pt=97 → 127.0.0.1:5002 → mesh → Pi port 5002 |
| Pi jitter buffer | rtpjitterbuffer latency=300 do-lost=true (the script default is 120; the WAN profile sets 300) |
| Pi decode | opusdec plc=true use-inband-fec=true |
| Pi processing | 48 kHz stereo S16LE → webrtcdsp: gain-control on (target-level-dbfs=6, i.e. −6 dBFS; compression-gain-db=6; limiter=true), high-pass-filter=true, noise-suppression=false, echo-cancel=false |
| Pi output | queue max-size-time=400 ms leaky=downstream → alsasink device=plughw:UAC2Gadget sync=false buffer-time=200000 latency-time=40000. The echo-cancel reference branch is built only when RETURN_AEC=1, and that is off |
| USB microphone (UAC2) | Stereo (p_chmask 3), 16-bit (p_ssize 2), offering 48000,44100,32000. The laptop opened 48 kHz stereo (period 1024, buffer 9216 frames) |

# 4. Audio: return, meeting laptop's speaker → Pi → Mac (unchanged)

| Stage | Setting |
| --- | --- |
| USB speaker (UAC2) | Stereo (c_chmask 3), 16-bit (c_ssize 2), c_sync adaptive, req_number 32 (tunable in /data/gadget-tuning.conf), offering 48000,44100,32000. The laptop opened 48 kHz stereo (period 960, buffer 9600 frames). Rate follow is on, with no mismatch |
| Pi capture | alsasrc device=hw:UAC2Gadget buffer-time=200000 latency-time=20000 at the laptop's rate (allowed rates 32000, 44100 and 48000), then queue max-size-time=300 ms leaky=downstream |
| Pi convert | audioresample quality=10 → 48 kHz stereo S16LE |
| Pi encoder | opusenc bitrate=128000 audio-type=generic inband-fec=true packet-loss-percentage=20 → rtpopuspay pt=97 → Mac port 5004. The app sets the destination at go-live |
| Mac jitter buffer | rtpjitterbuffer latency=250 do-lost=true |
| Mac playback | opusdec use-inband-fec=true (PLC off, concealment off) → audioresample quality=10 → volume 1.0 → queue max-size-time=400 ms → osxaudiosink sync=true buffer-time=200000 latency-time=20000 |

# 5. Pi system configuration (image 52a161b)

| Area | Setting |
| --- | --- |
| Boot, config.txt [all] | dtoverlay=dwc2,dr_mode=peripheral, dtoverlay=disable-bt, arm_freq=900, arm_boost=0, gpu_mem=16, dtparam=audio=off, activity and power LEDs off, kernel=kernel612.img, initramfs initramfs612 followkernel |
| Boot, cmdline.txt | modules-load=dwc2, overlayroot=tmpfs:recurse=0 (read-only root), root=PARTUUID=0d18cc81-02 |
| Card layout | p1 boot (vfat) · p2 rootA (committed slot) · p3 rootB (A/B standby) · p4 /data, grown to fill the card on first boot |
| Persistent binds on /data | tailscale state, /etc/bridge, NetworkManager connections, journal, diagnostics, /etc/default/bridge-return-audio, /etc/default/bridge-agent, /etc/default/bridge-net (the WAN seed), and /home/pi/flight.txt → /data/flight.txt |
| Journal | Storage=persistent, SystemMaxUse=200M, ReadKMsg=no. The -61 storm cost journald 30–37% of a core; kernel messages remain available in dmesg |
| Hardware watchdog | RuntimeWatchdogSec=15, RebootWatchdogSec=2min |
| Wi-Fi | NetworkManager power save off (wifi.powersave=2) and no MAC randomisation |
| Enabled services | bridge-gadget, bridge-feeder-net, bridge-uvcd, bridge-feeder-audio, bridge-return-audio, wifi-guardian, bridge-powertrim, flight-recorder, jitter-sentry, bridge-supervisor, bridge-web, bridge-wifi-portal, bridge-idle-frame, gadget-clean-detach, bridge-ab-healthcheck, bridge-wifi-unblock, bridge-firstdiag, bridge-firstboot, bridge-regen-hostkeys, bridge-identity, tailscaled; timers bridge-agent (15 s) and bridge-watchdog (5 min backstop) |
| Deliberately off | bridge-testpattern, bridge-crackle-sentry, bridge-pitch, bridge-idle-frame.timer (the frame never changes). Stock units masked: userconfig, cloud-init*, rpi-resize, systemd-growfs-root |
| Network watcher (jitter-sentry) | Never switches to the LAN profile (owner's rule). It moves to WAN only when the audio buffer is under 300 ms and the network is bad, so it does nothing on this WAN-300 card. Switches wait until media is idle. The live buffer escalation (JITTER_SENTRY_LIVE_RUNG) is off |
| Flight recorder | Python, one line per second: HH:MM:SS up= udc= thr= pull=. The last 500 lines are kept; each line is fdatasync'd, and pull=1 means the camera pump reported within 4 s |
| Status page | bridge-web on port 8080. The whole answer is rebuilt at most every 2 s (_STATUS_TTL); vcgencmd runs at most once a second; expected fps is read from the USB descriptor |
| Fleet agent | Heartbeat and command pull every 15 s to https://fleet.scine.online |
| Signed script updates | Four scripts can be overridden: bridge-feeder-net.sh, bridge-feeder-audio.sh, bridge-uvcd.sh, bridge-return-audio.sh. The signature is verified at install and at every start against /etc/netbridge/script-pubkey.pem on the read-only root. Failures are quarantined and the built-in script is used instead. Since 6f8a60a, deploys sync to the card before restarting |
| USB identity | VID 0x0525, PID 0xa4a2, serial 0123456789 (shared by all bridges, see §10), manufacturer = hostname, product "UVC Gadget", bcdUSB 0x0200, composite class 0xEF/0x02/0x01 |

# 6. Live verification

| Check (20:34 / 20:49 IST) | Result |
| --- | --- |
| Streams | Video, voice and return live; 0 restarts on every media service |
| Video to the laptop | 30 fps steady (59–61 frames per 2 s) |
| Mac → Pi packets | 220,272 video and 129,607 voice packets, 0 gaps |
| Wi-Fi Mac ↔ Pi | Round trip averaged 5.3 ms, worst 10.7 ms |
| Pi | CPU about 40% busy, load about 2, temperature 46 °C, uptime 57 min at 20:49, nothing quarantined |
| Audio | Clock "clean"; return at 48 kHz, no mismatch |
| USB camera misses | ⚠️ 6.6–12 per second (not visible now) |
| Power | ⚠️ Under-voltage 57–67% of the time |

# 7. Estimated latency

| Path | Estimate |
| --- | --- |
| Video: Mac camera → meeting | About 0.21–0.28 s. Camera frame ~33 ms, encode ~5–10, Wi-Fi ~3, jitter buffer 100, decode ~6, hand-off 0–33, USB ~25, laptop display ~33–66 |
| Voice: Mac mic → meeting | About 0.4–0.6 s (300 ms jitter buffer plus device buffers) |
| Return: meeting → Mac | About 0.35–0.7 s (250 ms jitter buffer plus device buffers) |

Voice probably trails the picture by about 0.2–0.3 s. It has not been reported as a problem and was left as is.

# 8. What changed 22–24 Sep and why (branch opt/pi-2026-09-22)

| Commit | Change | Why |
| --- | --- | --- |
| 083c725 | App: mesh auth key passed via the environment, not argv | The key was visible in ps |
| 40d0192 | Camera EAGAIN log filter; hardware H.264 decoder | The log flood; CPU (the decoder was later reverted) |
| e7487c7 | Status page cached | About 20 program launches per request, 25–33% of a core |
| 514277f | CPU sweep: Python flight recorder, agent reuses the status page, AEC reference branch only when AEC is on, idle-frame timer off | Work that produced nothing (the flight recorder alone was ~11% of a core) |
| 8434651 | journald ReadKMsg=no; software decoder; video buffer capped at 100 ms; gpu_mem=16; 4 s pull window | The -61 storm cost 30–37% of a core; the hardware path measured worse; the 300 ms buffer was the lag |
| d469ebd | App: H.264 Baseline | 41% less Pi decode CPU at the same quality |
| 37b61e7 / b771b47 | App: 800 kbps; test floor taken from measurement | Smaller keyframe bursts, less CPU |
| 98a6cde | 480×270 end to end, maxpacket 1024 | One USB packet per slot (superseded by f79ff06) |
| f79ff06 | 424×240 @ 30 fps end to end, 600 kbps; _expected_fps bug fixed | The camera's native rate (no judder), still one packet per slot (75%) |
| 52a161b | WAN profile seeded on /data; jitter-sentry never switches to LAN | No automatic media restarts on a fresh card; the owner's rule |
| 6f8a60a | USB-miss counter built into the camera service; deploys sync before restarting; this document | Make the verified setup the default |

Why the video froze (measured 2026-09-24). Everything upstream of the USB link was clean:

- the Mac camera delivered about 30 fps;

- there was 0 packet loss;

- Wi-Fi was at worst about 12 ms;

- the Pi decoded cleanly and handed 30 fps to the gadget.

The fault was on the USB link itself:

- The same freezes appeared in Windows' Camera app.

- The camera pump saw about 57–60 USB completions per second, against a physical maximum of about 40 full frames per second, so transfers were being cut short.

- The stock kernel cancels the whole UVC queue on a -61 miss.

WaysToGo reached the same conclusion on 16 Sep. Moving the IRQ to CPU 2 reduced the misses but did not remove them.

# 9. Operating rules

- Never switch a bridge to the LAN profile (bridge profile lan or the fleet profile command). Cut lag elsewhere.

- Unplug the meeting laptop before any camera-service (bridge-uvcd) restart or deploy. With a host attached, the USB controller hangs and the Pi reboots. This happened 2 out of 2 times on 2026-09-24, and it also explains the 22 Sep restart → reboot cases.

- Don't restart media services during a meeting. A restart of the video feeder alone, while the camera holds /dev/video40, crash-loops video. To recover: do a clean boot, or stop uvcd → restart the feeder → start uvcd.

- Signed updates.

- Run bash tools/publish-script.sh pi/scripts/<script>.

- Send the fleet command {"type":"deploy-script","args":{"name":"<script>","source":"https://fleet.scine.online/payloads"},"confirm":true}.

- Check with {"type":"running"}. The results of slow commands are sometimes lost.

- Ask before sending. A fleet command queued by a tool call runs even if that call is interrupted, so ask first, then send.

- Reading the USB-miss counter: {"type":"read-file","args":{"path":"/run/netbridge-usb-video.txt","lines":12,"tail":true}}.

# 10. Culprits and open issues (as of 20:49 IST)

| # | Issue | Impact | Next step |
| --- | --- | --- | --- |
| 1 | USB camera misses, 6.6–12/s in bursts | The source of the earlier freezes; they can come back with more motion or worse power | Test variant C (CPU 2 reserved for USB), variant B (IRQ on CPU 0), WaysToGo's kernel patch and an MJPEG prototype, measuring each with the counter |
| 2 | Power: under-voltage 57–67% of the time | USB misses; resets on camera restarts and at go-live | GPIO wiring (both 5 V pins 2 and 4, grounds 6 and 14, short thick wires) or the supply |
| 3 | main does not contain this configuration | A future build from main (for example by a collaborator) reverts to 640×360@20, which does not match app 1.4.3 | Merge opt/pi-2026-09-22 into main (the owner's decision) |
| 4 | The app's self-updater decides by file digest, not by version | It could install an OLDER build if one were published | Add a "never install a lower version" rule in the next app build |
| 5 | All bridges share USB serial 0123456789 | Windows caches per serial, so two bridges on one laptop conflict | Give each bridge its own serial (C-01) |
| 6 | The Mac return player logged 3 "timestamp discontinuity, resyncing" warnings in about 11 minutes | Possible brief skips in return audio | Watch; investigate only if heard |
| 7 | Slow fleet commands can lose their result | A deploy may look "timed out" when it succeeded, or the reverse | Verify with running; agent timeout fix (D2) |
| 8 | The test runner scores unittest-style files as "NO RESULT" | 10 passing test files look unscored, and the suite prints "NOT GREEN" | Teach tools/run-tests.sh to read unittest's OK / FAILED |

# 11. Restore or rebuild

- Restore this exact setup.

- Flash 2.1.0-52a161b (Desktop › NetBridge-Image, or release v2.1.0-52a161b).

- With the laptop unplugged, deploy the signed camera update 97219e9c. An image built from this branch already has it built in, so skip this step there.

- On the Mac, run ~/Desktop/NetBridge/Launch NetBridge.command (1.4.3).

- Keep the WAN profile.

- Build an image from this branch.

- Run gh workflow run build-image.yml --ref opt/pi-2026-09-22 -f draft=true.

- Verify both manifest signatures with the OTA public key.

- Run bash tools/image-audit.sh <image.img>.

- Build the Mac app. In app/netbridge-source, using the Python 3.11 venv that has PyInstaller, run python build.py --version 1.4.3 --signing-key ~/.netbridge/keys/app-signing-key.pem. The output goes to dist/ and is ad-hoc signed, for this Mac only.

- Run the tests. bash tools/run-tests.sh. Results on 2026-09-24:

  - 495 passed.

  - 10 unittest files pass (OK) but show as "NO RESULT" (culprit 8).

  - test-usb-audio-analysis needs numpy.

  - 2 fail. They fail the same way at main's merge base c4bfd89, so they are not caused by this branch:

    - test-fleet-drift: a comment-only match is flagged as security drift.

    - test-app-diagnosis: camera-release escalation order.

# 12. Where things are

| What | Where |
| --- | --- |
| This document (Word) | docs/NetBridge-Working-Configuration-2026-09-24.docx; the owner's copy is in ~/Downloads |
| Images | ~/Desktop/NetBridge-Image/ with audit reports: slot 1 is the latest (52a161b, in the Pi now), slots 2–4 are rollbacks |
| Mac app (default) | ~/Desktop/NetBridge/ (1.4.3); older builds in ~/Desktop/NetBridge/Older versions/ |
| Mac app logs | ~/.netbridge-source/logs/ (mesh gaps every 10 s per stream) |
| Pi overrides | /data/overrides/ (signed); quarantine in /data/overrides/quarantine |
| Pi USB-miss counter | /run/netbridge-usb-video.txt |
| Pi flight recorder | /home/pi/flight.txt → /data/flight.txt |
| Keys (Mac) | ~/.netbridge/keys/ (app, OTA and script signing keys; never in the repo) |